SECTION 32 39 13

MANUFACTURED METAL BOLLARDS

ALTERNATIVE SECTIONS 05 74 00 Decorative Metal Castings

** NOTE ** Edit this guide specification to meet project requirements.

FAAC Access Automation Business Unit is part of FAAC group.

Headquarters:

Via Calari, 10

40069 Zola Predosa, Bologna – Italy

Tel. +39 051 61724

Fax +39 051 75 85 18

www.faacgroup.com	

FAAC Group designs manufactures and distributes the most secure, innovative and functional solutions worldwide for every pedestrian and vehicular need: automatic access - parking - access control. FAAC Group satisfies the service and reliability needs of residential, commercial and industrial clients. FAAC Group makes the city move, creating automation that simplifies the lives of those who live there.

Today, after more than fifty years of business and thanks to steady growth in all areas of automation and access control, FAAC has turned into a large multinational industrial group with over 80 countries served by sales offices or official distributors.

- GENERAL

SECTION INCLUDES

Hydraulic Automatic Retractable Bollards:

Access Control - perimeter security application Hydraulic Automatic retractable bollards (Model JS 80 HA)

RELATED REQUIREMENTS

Section 02 22 19 – Traffic Assessment

Section 32 31 00 – Fences and Gates

Section 34 71 13 - Paving and Accessories

Section 03 30 00 - Cast-In-Place Concrete

REFERENCES

Submit under provisions of Section 01 42 00 – References

Abbreviations and Acronyms

F: Type bollard (Static)

REGULATORY REQUIREMENTS

** NOTE ** Delete the standard which are not applicable to your area

Submit under provisions of Section 01 41 00 – Regulatory Requirements

- PAS 68: Impact test specifications for vehicle security barrier systems

- IWA 14-1: Vehicle Security Barriers - Part 1: Performance requirement, vehicle impact test method and performance rating

- ASTM F2656: Standard Test Method for Vehicle Crash Testing of Perimeter Barriers

SUBMITTALS

Submit under provisions of Section 01 33 00 – Submittal Procedures

Product Data: Provide for each product equipment list, system description and manufacturer's data sheets, including:

Product information, manufacturer’s names and identification of product.

Component connections and details.

Storage and handling requirements and recommendations.

Shop Drawings: Submit shop drawings showing layout, product components, including anchorage, excavation details and accessories:

Drawings, including wiring diagrams.

Risers, layouts, hardware location and accessories predispositions.

Special wiring diagrams for optional configurations.

Manufacturer’s Instructions:

Preparation instructions and recommendations.

Operation and installation instructions and manuals.

Manufacturer’s Certificates: Certify products meets or exceed specified compliances

Maintenance Data: provide manufacturer's maintenance instructions that include:

Lists of periodic checking and maintenance programs

Spare parts lists

Equipment inter-connection diagrams

DELIVERY, STORAGE, AND HANDLING

Submit under provisions of Section 01 66 00 – Product Storage and Handling Requirements

Deliver, store and handle materials and products in strict compliance with manufacturer's instructions and industry standards.

Store products indoors in manufacturer's original containers and packaging, with labels clearly identifying product name and manufacturer. Protect from damage.

QUALITY ASSURANCE

Submit under provisions of Section 01 43 00 – Quality Assurance

Manufacturer Qualifications:

ISO 9001 Certified Manufacturer

Manufacturer with minimum of 50 year experience

Installer Qualifications: Installation performed by installer specifically trained in bollards operation systems of the type found within this section.

Furnish bollards, all components and accessories by one manufacturer.

SITE CONDITIONS

Submit under provisions of Section 01 71 00 – Examination and Preparation

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

Verify installation required dimensions before installing.

Ensure that excavating surface is appropriate for installation:

The characteristic of the ground must guarantee sufficient solidity of the foundation plinth.

No tubes or electrical cables should be present in the digging area.

WARRANTY

** NOTE ** Apply special warranty provided by local subsidiaries

Submit under provisions of Section 01 78 00 – Closeout Submittals

Manufacturer's Standard Limited Warranty:

Warranty Period: 2 years of free repair or replacement from date of installation

Special warranty extensions may apply

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: FAAC SpA, which is located at: Via Calari, 10 - 40069 Zola Predosa, Bologna – Italy / Tel: (+39)-051-61724 / Fax: (+39)-051-758518 /                     Email: info@faacgroup.com  /  Web: www.faacgroup.com

Substitutions: Not permitted

Requests for substitutions will be considered in accordance with provisions of Section 01 25 00 - Substitution Procedures.

Model JS 80 HA

Overall Description: Hydraulic Automatic retractable bollard for perimeter security applications. Thanks to the certified crash resistance, the product is targeted for the traffic control and the perimeter protection of sensitive areas.

Drive

Hydraulic power unit (HPU)

Power supply: 207 V~ ... 243 V~ _ 50/60 Hz

Max Power consumption: 2.700 W

Protection Class: IP67 (HPU)

Cylinder

Height from ground: 1000 mm  / 40 inches

Diameter: 275 mm / 11 inches

Material

Body: high performance steel

Sleeve: mDure® protective sleeve

Head: Aluminium with Rilsan® resin coating

Colour

Body: Dark grey with FAAC design

Head: Black

Reflective strip

Height: 55 mm / 2.2 inches

Colour: White

Suggested Usage: Perimeter protection

Resistance Force: Break-in: 1.852.000 J

- according to:

- - PAS 68: Impact test specifications for vehicle security barrier systems

- - IWA 14-1: Vehicle Security Barriers - Part 1: Performance requirement, vehicle impact test method and performance rating

- - ASTM F2656: Standard Test Method for Vehicle Crash Testing of Perimeter Barriers

Performances

      - Operating Time

        - Rising speed: 170 mm/s

        - Lowering speed: 250 mm/s

Ambient Temperature range

Standard: 	-15°C + 80°C / 5°F + 176°F

With Heater: 	-40°C + 80°C / -40°F + 176°F (optional accessory)

Features

Weight: 670kg / 1.480 pounds

Model JS 80 HA STAINLESS STEEL

Overall Description: Hydraulic Automatic retractable bollard for perimeter security applications. Thanks to the certified crash resistance, the product is targeted for the traffic control and the perimeter protection of sensitive areas.

Drive

Hydraulic power unit (HPU)

Power supply: 207 V~ ... 243 V~ _ 50/60 Hz

Max Power consumption: 2.700 W

Protection Class: IP67 (HPU)

Cylinder

Height from ground: 1000 mm  / 40 inches

Diameter: 275 mm / 11 inches

Material

Body: high performance steel

Sleeve: mDure® protective sleeve

Head: Aluminium with Rilsan® resin coating

Colour

Body: Silver with satin finishing

Head: Black

Reflective strip

Height: 55 mm / 2.2 inches

Colour: White

Suggested Usage: Perimeter protection

Resistance Force: Break-in: 1.852.000 J

- according to:

- - PAS 68: Impact test specifications for vehicle security barrier systems

- - IWA 14-1: Vehicle Security Barriers - Part 1: Performance requirement, vehicle impact test method and performance rating

- - ASTM F2656: Standard Test Method for Vehicle Crash Testing of Perimeter Barriers

Performances

      - Operating Time

        - Rising speed: 170 mm/s

        - Lowering speed: 250 mm/s

Ambient Temperature range

Standard: 	-15°C + 80°C / 5°F + 176°F

With Heater: 	-40°C + 80°C / -40°F + 176°F (optional accessory)

Features

Weight: 670kg / 1.480 pounds

Model JS 80 HA EFO EMERGENCY FAST OPERATION)

Overall Description: Hydraulic Automatic retractable bollard for perimeter security applications with option for an emergency quick rise of the cylinder . Thanks to the certified crash resistance, the product is targeted for the traffic control and the perimeter protection of sensitive areas.

Drive

Hydraulic power unit (HPU)

Power supply: 207 V~ ... 243 V~ _ 50/60 Hz

Max Power consumption: 4.000 W

Protection Class: IP67 (HPU)

Cylinder

Height from ground: 1000 mm  / 40 inches

Diameter: 275 mm / 11 inches

Material

Body: high performance steel

Sleeve: mDure® protective sleeve

Head: Aluminium with Rilsan® resin coating

Colour

Body: Dark grey with FAAC design

Head: Black

Reflective strip

Height: 55 mm / 2.2 inches

Colour: White

Suggested Usage: Perimeter protection

Resistance Force: Break-in: 1.852.000 J

- according to:

- - PAS 68: Impact test specifications for vehicle security barrier systems

- - IWA 14-1: Vehicle Security Barriers - Part 1: Performance requirement, vehicle impact test method and performance rating

- - ASTM F2656: Standard Test Method for Vehicle Crash Testing of Perimeter Barriers

Performances

      - Operating Time

        - Rising speed: 170 mm/s

        - Rising speed with EFO activation: 500 mm/s

        - Lowering speed: 250 mm/s

Ambient Temperature range

Standard: 	-15°C + 80°C / 5°F + 176°F

With Heater: 	-40°C + 80°C / -40°F + 176°F (optional accessory)

Features

Weight: 700kg / 1.540 pounds

Model JS 80 HA STAINLESS STEEL EFO EMERGENCY FAST OPERATION)

Overall Description: Hydraulic Automatic retractable bollard for perimeter security applications with option for an emergency quick rise of the cylinder . Thanks to the certified crash resistance, the product is targeted for the traffic control and the perimeter protection of sensitive areas.

Drive

Hydraulic power unit (HPU)

Power supply: 207 V~ ... 243 V~ _ 50/60 Hz

Max Power consumption: 4.000 W

Protection Class: IP67 (HPU)

Cylinder

Height from ground: 1000 mm  / 40 inches

Diameter: 275 mm / 11 inches

Material

Body: high performance steel

Sleeve: mDure® protective sleeve

Head: Aluminium with Rilsan® resin coating

Colour

Body: Silver with satin finishing

Head: Black

Reflective strip

Height: 55 mm / 2.2 inches

Colour: White

Suggested Usage: Perimeter protection

Resistance Force: Break-in: 1.852.000 J

- according to:

- - PAS 68: Impact test specifications for vehicle security barrier systems

- - IWA 14-1: Vehicle Security Barriers - Part 1: Performance requirement, vehicle impact test method and performance rating

- - ASTM F2656: Standard Test Method for Vehicle Crash Testing of Perimeter Barriers

Performances

      - Operating Time

        - Rising speed: 170 mm/s

        - Rising speed with EFO activation: 500 mm/s

        - Lowering speed: 250 mm/s

Ambient Temperature range

Standard: 	-15°C + 55°C / 5°F + 131°F

With Heater: 	-40°C + 55°C / -40°F + 131°F (optional accessory)

Features

Weight: 700kg / 1.540 pounds

CONTROL UNIT

** NOTE ** Add eventual extra accessories

Provide the Control Unit: Model JE275 integrated controller with 2 digits display, 8 digital inputs, 4  digital outputs (one equipped with a relay)

Integrated Loop Detectors: 2 double channel loop detectors, with high sensibility

Digital inputs:

Input voltage: 24 VDC

Total accessories max current: 500 mA

Digital outputs: 2 open collector

Max switching voltage 24 VDC

Max switching current 100 mA

Output relay: 1 normally open contact (isolated)

Max switching voltage: 24 VDC

Max switching current: 500 mA

Maximum 8 pairs of bus photocells connectable

Quick plugin connectors: BUS, radio receiver / decoding card

Cycles counter

Most I/O parameters are adjustable

Most performances are adjustable: deceleration, torque, times.

10 pre-set adjustable operating logics

Master/Slave configurations option available

Display: 2 digit 7 segment LED 0,762 cm / 0.3 inches red segments.

ACCESSORIES

** NOTE ** Add eventual extra accessories

Provide the required installation complement  listed below:

BOLLARD PIT JS HA PIT

Provide the optional accessories listed below:

Pit heater: to enlarge the operation temperature range to extreme low temperatures (as above specified).

Emergency hand pump: allows to rise the cylinder in case of blackout.

- EXECUTION

EXAMINATION AND PREPARATION

Verify that all site conditions, installation and operating conditions are compliant to the sections and manufacturer’s requirements express in the instructions.

Inspect and prepare substrates and surface properly in accordance to the manufacturer’s recommendations.

If preparation is the responsibility of another installer, notify Architect in writing of deviations from manufacturer's recommended installation tolerances and conditions.

Clean surfaces thoroughly prior to installation.

Verify that required drainage and electrical services are in place as required for the installation.

Verify that the product surrounding area is clear from obstacle and flying electric wiring for proper beam motion

It is suggested to provide an adequate protection against accidental impact if product will be exposed to vehicles, when possible

INSTALLATION

Foundation

Required excavation dimensions: 1.600 mm x 3.100 mm  x 1.700 mm // 63 inches x 122 inches x 67 inches* (wide x deep x tall)

- * add 10mm / 0.4 inches above the foundation to lay the pavement and 10mm / 0.4 inches below for the base of mud slab

Concrete Specifications: Class C25/30 Concrete with 10-30 aggregate according to EN 12620 standard. Bollard shall be installed after at least 7 days of concrete setting

Surrounding ground compacting index: ≥ 90% of the  Proctor optimum curve, according to EN 13286-2:2005 standard

Install in accordance with manufacturer's instructions. Test for proper operation and adjust until satisfactory results are obtained.

Clean all metal surfaces promptly after installation.

FINAL ADJUSTMENT

Check safety devices and compliance to safety regulations, otherwise adjust parameters or replace defective parts until satisfactory results are obtained.

PROTECTION

Protect installed products until completion of project.

Prevent product from damage.

Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION